# Scope

This specification describes the OPC Unified Architecture (OPC UA) Profiles. The Profiles in this part are used to describe the functionality that an OPC UA Server exposes or that an OPC UA Client consumes. The details of the functionality are specified in other part of the OPC UA specification.

Profiles are used by vendors to advertise the OPC UA capabilities of their products. The Profiles a product supports will typically appear on product data sheets. Buyers will use this Profile information to specify and purchase products that work together and meet specific application requirements. Most OPC UA applications will conform to several, but not all of the Profiles.

A product’s conformance to a Profile is measured using self-testing tools (Compliance Test Tools) and via Independent Certification Test Labs. Some Profiles describe functionality that is only testable in a test lab environment. These Profiles will clearly state this in their description.

# Reference documents

Part 1 : OPC UA Specification: Part 1 – Concepts, Version 1.01 or later.

http://www.opcfoundation.org/UA/Part1/

Part  2 : OPC UA Specification: Part 2 – Security Model, Version 1.01 or later

http://www.opcfoundation.org/UA/Part2/

Part  3 : OPC UA Specification: Part 3 – Address Space Model, Version 1.01 or later

http://www.opcfoundation.org/UA/Part3/

Part  4 : OPC UA Specification: Part 4 – Service, Version 1.01 or later

http://www.opcfoundation.org/UA/Part4/

Part  5 : OPC UA Specification: Part 5 – Information Model, Version 1.01 or later

http://www.opcfoundation.org/UA/Part5/

Part  6 : OPC UA Specification: Part 6 – Mapping, Version 1.01 or later

http://www.opcfoundation.org/UA/Part6/

Part  8 : OPC UA Specification: Part 8 – Data Access, Version 1.01 or later

http://www.opcfoundation.org/UA/Part8/

Part  9 : OPC UA Specification: Part 9 – Alarms and Conditions, Version 1.01 or later

http://www.opcfoundation.org/UA/Part9/

Part  10 : OPC UA Specification: Part 10 – Programs, Version 1.01 or later

http://www.opcfoundation.org/UA/Part10/

Part  11 : OPC UA Specification: Part 11 – Historical Access, Version 1.01 or later

http://www.opcfoundation.org/UA/Part11/

Part  12 : OPC UA Specification: Part 12 – Discovery, Version 1.01 or later

http://www.opcfoundation.org/UA/Part12/

Part  13 : OPC UA Specification: Part 12 – Discovery, Version 1.01 or later

http://www.opcfoundation.org/UA/Part12/

Test Specifications

Compliance Part 8 : OPC Test Lab Specification: Part 8 – UA Server

http://www.opcfoundation.org/UA/Part12/

Compliance Part 9 : OPC Test Lab Specification: Part 9 – UA Client

http://www.opcfoundation.org/UA/Part12/

Compliance Part 10] : OPC Test Lab Specification: Part 10 – UA Abstract Test

http://www.opcfoundation.org/UA/Part12/

Kerberos  Token :  wss-v1.1-spec-os-KerberosTokenProfile

http://www.oasis-open.org/committees/download.php/16788/wss-v1.1-spec-os-KerberosTokenProfile.pdf

# Terms, definitions, and conventions

## OPC UA part 1 terms

The following terms defined in Part 1 apply.

- AddressSpace

- Alarm

- Attribute

- Certificate

- Client

- Complex Data

- Condition

- Discovery

- Event

- EventNotifier

- Message

- Method

- MonitoredItem

- Node

- NodeClass

- Notification

- Object

- Object Type

- Profile

- Program

- Reference

- ReferenceType

- Server

- Service

- Service Set

- Session

- Subscription

- Variable

- View

## OPC UA part 2 terms

The following terms defined in Part  2 apply.

- Asymmetric Encryption

- Asymmetric Signature

- Auditing

- Authentication

- Authorization

- Message Signature

- Private Key

- Public Key

- Public Key Infrastructure (PKI)

- Rivest-Shamir-Adleman (RSA)

- Secure Channel

- Symmetric Cryptography

- Symmetric Encryption

- Symmetric Signature

- X.509 Certificate

## OPC UA part 3 terms

The following terms defined in Part  3 apply.

- DataVariable

- EventType

- HierarchicalReference

- ModellingRule

- Property

- VariableType

## OPC UA part 4 terms

The following terms defined in Part  4 apply.

- Deadband

- EndPoint

- SoftwareCertificate

## OPC UA part 6 terms

The following terms defined in Part  6 apply.

- Data Encoding

- Stack Profile

- Transport Protocol

## OPC UA part 8 terms

The following terms defined in Part  8 apply.

- AnalogItem

- DataItem

- DiscreteItem

- EngineeringUnits

## OPC UA Profile terms

### ConformanceUnit

A specific set of features (e.g. a group of services, portions of services or information models) that can be tested as a single entity

## Abbreviations and symbols

API	Application Programming Interface

DA	Data Access

HA	Historical Access

HMI	Human Machine Interface

UA 	Unified Architecture

# Overview

## General

The OPC UA multipart specification describes a number of Services and a variety of information models. These Services and information models can be referred to as features of a server or client. Servers and Clients need to be able to describe which of the features they support. This part provides a grouping of these features. The individual features are grouped into ConformanceUnits which are further grouped into Profiles.  Figure 1 provides an overview of the interactions between Profiles, ConformanceUnits and test cases.  The figure also illustrates a feature of the OPC UA compliance test tool, in that it will test if a requested Profile passes all ConformanceUnits. It will also test all other ConformanceUnits and report any other Profiles that pass conformance testing.

Figure 1 - Profile - ConformanceUnit - Test cases

## ConformanceUnit

Each ConformanceUnit represents a specific set of features (e.g. a group of services, portions of services or information models) that can be tested as a single entity. ConformanceUnits are the building blocks of a Profile. For each ConformanceUnit there are a number of test cases that test the functionality described in the ConformanceUnit. The description of a ConformanceUnit is intended to provide enough information to illustrate the required functionality, but in many cases to obtain a complete understanding of the ConformanceUnit the reader may be required to also examine either the appropriate part of the OPC UA specification or the details provided in the OPC UA Client or OPC UA Server test specification.

The same features do not appear in more than one ConformanceUnit.

## Profiles

Profiles are named groupings of ConformanceUnits. The Servers and Clients in an OPC UA application will provide the names of Profiles that they support. The definition of Profiles is a dynamic activity, in that it is expected that new Profiles will be added in the future. A Profile can be defined to inherit from an existing Profile. The new Profile may add additional ConformanceUnits. These additional ConformanceUnits may add additional features that are to be tested. The additional ConformanceUnits may also further restrict inherited ConformanceUnits.

An OPC UA application will typically support multiple Profiles.

Multiple Profiles may include the same ConformanceUnit.

Testing of a Profile consists of testing the individual ConformanceUnits that comprise the Profile.

## Profile Categories

Profiles are grouped into categories to help vendors and end users understand the applicability of a Profile.  Categories include: Server and Client, but they could also include Power Generation or Chemical Plant. A Profile can be assigned to more than one category. Table 1 – Profiles Categories contains the list of defined Profile categories.

Table 1 – Profiles Categories

| Category | Description |
| --- | --- |
| Client | Profiles of this category specify a complete functional set for an OPC UA Client. The URI of such profiles has to be part of a Software Certificate passed in the CreateSession request. |
| Security | Profiles of this category specify a security policy. The URI of such profiles has to be part of an Endpoint Description returned from the GetEndpoint service. |
| Server | Profiles of this category specify a complete functional set for an OPC UA Server. The URI of such profiles has to be part of a Software Certificate returned with the CreateSession service response. |
| Transport | Profiles of this category specify a specific protocol mapping. The URI of such profiles has to be part of an Endpoint Description. |

# ConformanceUnits

## Overview

A ConformanceUnit represents an individually testable entity. The list of ConformanceUnits is large and as a result ConformanceUnits are combined for easier understanding into named conformance groups. These groups closely correspond to the Service Sets in Part  4 and to the OPC UA information models. Table 2 provides a list of the conformance groups. These groups and the ConformanceUnits that they describe are detailed in the following sections. Conformance Groups have no impact on compliance testing; they are used only for organizational reasons in this document.

Table 2 - Conformance Groups

| Group | Description |
| --- | --- |
| Address Space Model | Defines ConformanceUnits for various features of the OPC UA AddressSpace. |
| Alarms & Conditions | All ConformanceUnits that are associated with the enhanced OPC UA information model for Conditions, acknowledgeable Conditions, confirmations and Alarms. |
| Attribute Services | Includes ConformanceUnits to Read or Write current or historical Attribute values. |
| Auditing | User level security includes support for security audit trails, with traceability between Client and Server audit logs. |
| Base Information | All information elements as defined in Part 5 of the OPC UA specification. |
| Data Access | ConformanceUnits specific to Clients and Servers that deal with the representation and use of automation data as specified in Part 8 of the OPC UA specification. |
| Discovery Services | ConformanceUnits with focus on Server Endpoint Discovery |
| Historical Access | Access to archived data of node Attribute values or Events. |
| Method Services | Methods represent the function calls of Objects. Methods are invoked and return only after completion (successful or unsuccessful). |
| Monitored Item Services | Clients define MonitoredItems to subscribe to data and Events. Each MonitoredItem identifies the item to be monitored and the Subscription to use to send Notifications. |
| Node Management Services | Bundles ConformanceUnits for all Services to add and delete AddressSpace Nodes and References. |
| Programs | Programs are complex functions in a server or underlying system that can be invoked and managed by an OPC UA Client. Programs can represent any level of functionality within a system or process in which client control or intervention is required and progress monitoring is desired |
| Protocol and Encoding | Covers all transport and encoding combinations that are specified in Part 6 of the OPC UA specification. |
| Query Services | A Query may be used to return a subset of data from the View. |
| Redundancy | The design of OPC UA ensures that vendors can create redundant Clients and redundant Servers in a consistent manner. Redundancy may be used for high availability, fault tolerance and load balancing. |
| Security | Security related ConformanceUnits that can be profiled. |
| Session Services | An (OPC UA) Session is an application layer connection. |
| Subscription Services | Subscriptions are used to report Notifications to the Client. |
| View Services | Clients use the View Service Set to navigate through the AddressSpace or through a View as subset of the AddressSpace. |

## Services

The following tables describe ConformanceUnits for the Services specified in Part  4. The tables correlate with the Service Sets.

A single ConformanceUnit can reference several Services (e.g. CreateSession, ActivateSession and CloseSession) but can also refer to individual aspects of Services (e.g. “The use of ActivateSession to impersonate a new user).

Each table includes a listing of the category to which a ConformanceUnit belongs, the title and description of the ConformanceUnit and a column that indicates if the ConformanceUnit is derived from another ConformanceUnit.  A ConformanceUnit that is derived from another ConformanceUnit includes all of the same tests as its parent plus one or more additional test cases.  These test cases can only further restrict the existing test cases.   An example would be one in which the number of connections is tested, where the test case of the parent required at least one connection and the derived ConformanceUnit would require a test case for at least five connections.

The Discovery Service Set is composed of multiple ConformanceUnits (see Table 3). All Servers provide some aspects of this functionality; see Profiles categorized as Server Profiles for details. Clients may support some aspects of this functionality; see Profiles categorized as Client Profiles for details.

Table 3 - Discovery Services

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Discovery Get Endpoints | Support the GetEndpoints Service to obtain all Endpoints of one Server. This includes filtering based on Profiles. |  |
| Server | Discovery Find Servers Self | Support the FindServers Service only for itself. |  |
| Server | Discovery Register | Call the RegisterServers Service to register itself (OPC UA Server) with an external Discovery Service. |  |
| Server | Discovery Register Security | Call the RegisterServers Service to register itself (OPC UA Server) with an external Discovery Service via a secure channel with a securityMode other than "None". |  |
| Server | Discovery Configuration | Allow configuration of the Discovery Server URL where the Server will register itself. Allow complete disabling of registration with a Discovery Server. |  |
| Client | Discovery Client Find Servers Basic | Uses the FindServers Service to obtain all Servers installed on a given node. |  |
| Client | Discovery Client Find Servers with URI | Use FindServers Service to obtain URLs for specific Server URIs. |  |
| Client | Discovery Client Find Servers Dynamic | Detect new Servers after an initial FindServers Service call. |  |
| Client | Discovery Client Get Endpoints Basic | Uses the GetEndpoints Service to obtain all Endpoints for a given Server URI. |  |
| Client | Discovery Client Get Endpoints Dynamic | Detect changes to the Endpoints after an initial GetEndpoints Service call. |  |
| Client | Discovery Client Configure Endpoint | Allow specification of an Endpoint without going through the Discovery Service Set. |  |

The Session Service Set is composed of multiple ConformanceUnits (see Table 4). The CreateSession, ActivateSession, and CloseSession services are supported as a single unit. All servers and clients provide this functionality.

Table 4 – Session Services

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Session General Service Behaviour | Implement basic Service behaviour. This includes in particular: - checking the authentication token - returning the requestHandle in responses - returning available diagnostic information as requested with the 'returnDiagnostics' parameter - respecting a timeoutHint |  |
| Server | Session Base | Support the Session Service Set (CreateSession, ActivateSession, CloseSession) except the use of ActivateSession to change the Session user. This includes correct handling of all parameters that are provided. Note that for the CreateSession and ActivationSession services, if the SecurityMode = None then: 1) The Application Certificate and Nonce are optional. 2) The signatures are null/empty. |  |
| Server | Session Cancel | Support the Cancel Service to cancel outstanding requests. |  |
| Server | Session Minimum 10 Parallel | Support minimum 10 parallel Sessions (total for all Clients). |  |
| Server | Session Minimum 50 Parallel | Support minimum 50 parallel Sessions (total for all Clients). |  |
| Client | Session Client General Service Behaviour | Implement basic Service behaviour. This includes in particular: - including the proper authentication token of the Session - creating requestHandles if needed - requesting diagnostic information with the 'returnDiagnostics' parameter - evaluate the serviceResult and operational results |  |
| Client | Session Client Base | Use the Session Service Set (CreateSession, ActivateSession, CloseSession) except the use of ActivateSession to change the Session user. This includes correct handling of all parameters that are provided Note that for the CreateSession and ActivationSession services, if the SecurityMode = None then: 1) The Application Certificate and Nonce are optional. 2) The signatures are null/empty. |  |
| Client | Session Client Renew NodeIds | This ConformanceUnit applies to Clients that persist NodeIds. Verify that the Namespace Table has not changed for NodeIds that the Client has persisted and is going to re-use beyond a Session lifetime. If changes occurred the Client has to recalculate the Namespace Indices of the respective NodeIds. |  |
| Client | Session Client Impersonate | Use of ActivateSession to change the Session user (impersonation). |  |
| Client | Session Client KeepAlive | Make periodic requests to keep the Session alive. |  |
| Client | Session Client Detect Shutdown | Read or monitor the ServerStatus/State Variable to recognize a potential shutdown of the Server and clean up resources. |  |
| Client | Session Client Cancel | Use the Cancel Service to cancel outstanding requests. |  |
| Client | Session Client Auto Reconnect | Automatic Client reconnect including: - ActivateSession with new SecureChannel if SecureChannel is not longer valid but Session is still valid - Creation of a new Session only if Session is not longer valid |  |

The Node Management Service Set is composed of multiple ConformanceUnits (see Table 5). Servers may provide some aspects of this functionality, see Profiles categorized as server Profiles for details. Clients may support some aspects of this functionality, see Profiles categorized as client Profiles for details.

Table 5 – Node Management Services

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Node Management Add Node | Support the AddNodes service to add one or more Nodes into the AddressSpace. |  |
| Server | Node Management Delete Node | Support the DeleteNodes Service to delete one or more Nodes from the AddressSpace. |  |
| Server | Node Management Add Ref | Support the AddReferences Service to add one or more References to one or more Nodes. |  |
| Server | Node Management Delete Ref | Support the DeleteReferences Service to delete one or more References of a Node. |  |
| Client | Node Management Client | Use Node Management Services to add or delete Nodes and to add or delete References in an OPC UA Server's AddressSpace. |  |

The View Service Set is composed of a multiple ConformanceUnits (see Table 6). All Servers support some aspects this conformance group.

Table 6 – View Services

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | View Basic | Support the View Service Set (Browse, BrowseNext). |  |
| Server | View TranslateBrowsePath | Support TranslateBrowsePathsToNodeIds Service. |  |
| Server | View RegisterNodes | Support the RegisterNodes and UnregisterNodes Services as a way to optimize access to repeatedly used Nodes in the Server's AddressSpace. |  |
| Server | View Minimum Continuation Point 01 | Support minimum 1 continuation point per Session. |  |
| Server | View Minimum Continuation Point 05 | Support minimum 5 continuation points per Session. | View Minimum Continuation Point 01 |
| Client | View Client Basic Browse | Use Browse and BrowseNext Services to navigate through the Server's AddressSpace. Make use of the referenceTypeId and the nodeClassMask to specify the needed References. |  |
| Client | View Client Basic ResultSet Filtering | Make use of the resultMask parameter to optimize the result set to be returned by the Server. |  |
| Client | View Client TranslateBrowsePath | Use the TranslateBrowsePathsToNodeIds Service to identify the NodeIds for Nodes where a starting Node and a BrowsePath is known. Make bulk operations rather than multiple calls whenever possible. |  |
| Client | View Client RegisterNodes | Use the RegisterNodes Service to optimize access for Nodes that are used repeatedly. Use UnregisterNodes when Nodes are not used anymore. |  |

The Query Service Set is composed of multiple ConformanceUnits (see Table 7). Servers may provide some aspects of this functionality, see Profiles categorized as server Profiles for details. Clients may support some aspects of this functionality, see Profiles categorized as client Profiles for details.

Table 7 – Query Services

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
|  | Undefined | No Conformance Units defined |  |

The Attribute Service Set is composed of multiple ConformanceUnits (see Table 8). The majority of the Attribute service set is a core functionality of the OPC UA specification.

Table 8 – Attribute Services

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Attribute Read | Support the Read Service to read one or more Attributes of one or more Nodes. This includes support of the IndexRange parameter to read a single element or a range of elements when the Attribute value is an array |  |
| Server | Attribute Read Complex | Support reading and encoding ComplexData (structures). |  |
| Server | Attribute Write Values | Support the Write Service to write values to one or more Attributes of one or more Nodes. |  |
| Server | Attribute Write Complex | Support writing and decoding ComplexData. |  |
| Server | Attribute Write StatusCode & TimeStamp | Support writing of StatusCode and Timestamps. |  |
| Server | Attribute Write Index | Support the IndexRange to write a single element or a range of elements when the Attribute value is an array. |  |
| Server | Attribute Alternate Encoding | Support alternate Data Encoding when reading value Attributes. By default, every Server has to support the Data Encoding of the currently used Stack Profile (i.e. binary with UA Binary Encoding and XML with XML Encoding). This ConformanceUnit - when supported - specifies that the other Data Encoding is supported in addition. |  |
| Client | Attribute Client Read Base | Use the Read Service to read one or more Attributes of one or more Nodes. This includes use of an IndexRange to select a single element or a range of elements when the Attribute value is an array. Clients shall use bulk operations whenever possible to reduce the number of Service invocations. |  |
| Client | Attribute Client Read with proper Encoding | This ConformanceUnit refers to the ability of a Server to support more than one Data Encoding for Attribute values. Clients can discover the available encodings and can explicitely choose one when calling the Read Service. |  |
| Client | Attribute Client Read Complex | Read and decode ComplexData (structures). |  |
| Client | Attribute Client Write Base | Use the Write Service to write values to one or more Attributes of one or more Nodes. This includes use of an IndexRange to select a single element or a range of elements when the Attribute value is an array. Clients shall use bulk operations whenever possible to reduce the number of Service invocations. |  |
| Client | Attribute Client Write Complex | Write and Encode ComplexValues (structures). |  |
| Client | Attribute Client Write Quality & TimeStamp | Use the Write Service to also write StatusCode and/or Timestamps. |  |

The Method Service Set is composed of ConformanceUnits (see Table 9). The primary ConformanceUnits provide support for the call functionality. Servers may provide some aspects of this functionality, see Profiles categorized as server Profiles for details. Clients may support some aspects of this functionality, see Profiles categorized as client Profiles for details.

Table 9 – Method Services

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Method Call | Support the Call Service to call (invoke) a Method. |  |
| Client | Method Client Call | Use the Call Service to call one or several Methods. |  |

The MonitoredItem Service Set is composed of multiple ConformanceUnits (see Table 10). Servers may provide some aspects of this functionality; see Profiles categorized as Server Profiles for details. Clients may support some aspects of this functionality, see Profiles categorized as client Profiles for details.

Table 10 – Monitored Item Services

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Monitor Basic | Support the following MonitoredItem Services: CreateMonitoredItems, ModifyMonitoredItems, DeleteMonitoredItems, SetMonitoringMode. |  |
| Server | Monitor Value Change | Support creation of MonitoredItems for Attribute value changes. This includes support of the IndexRange to select a single element or a range of elements when the Attribute value is an array. |  |
| Server | Monitor Alternate Encoding | Support alternate encoding when monitoring value Attributes. By default, every Server has to support the encoding of the currently used Stack Profile ( i.e. binary with UA Binary Encoding and XML with XML Encoding). This ConformanceUnit - when supported - specifies that the other encoding is supported in addition. |  |
| Server | Monitor Items 10 | Support at least 10 MonitoredItems per Subscription. |  |
| Server | Monitor Items 100 | Support at least 100 MonitoredItems per Subscription. | Monitor Items 10 |
| Server | Monitor Items 500 | Support at least 500 MonitoredItems per Subscription. | Monitor Items 100 |
| Server | Monitor QueueSize_1 | This ConformanceUnit does not require queueing when multiple value changes occur during a publish period. I.e. the latest change will be sent in the Notification. |  |
| Server | Monitor MinQueueSize_02 | Support at least 2 queue entries for MonitoredItems. Servers often will adapt the queue size to the number of currently MonitoredItems. However, it is expected that Servers support this minimum queue size for at least one third of the supported MonitoredItems. |  |
| Server | Monitor Triggering | Support the SetTriggering Service to create and/or delete triggering links for a triggering item. |  |
| Server | Monitor Events | Support creation of MonitoredItems for a "Notifier Attribute" for the purpose of Event Notification. |  |
| Client | Monitor Client Value Change | Use the MonitoredItem Service Set to register items for changes in Attribute value. Use CreateMonitoredItems to register the Node/Attribute tuple. Set proper sampling interval, Deadband filter and queueing mode. Use disabling / enabling instead of deleting and re-creating a MonitoredItem. Use bulk operations rather than individual service requests to reduce communication overhead. |  |
| Client | Monitor Client by Index | Use the IndexRange to select a single element or a range of elements when the Attribute value is an array. |  |
| Client | Monitor Client Events | Use the MonitoredItem Service Set to create MonitoredItems for Event notifications. |  |
| Client | Monitor Client Event Filter | Use the Event filter when calling CreateMonitoredItems to filter the desired Events and to select the columns to be provided for each Event Notification. |  |
| Client | Monitor Client Complex Evt Filter | Uses complex Event filters | Monitor Client Event Filter |
| Client | Monitor Client Modify | Use ModifyMonitoredItems Service to change the configuration setting. Use SetMonitoringMode Service to disable / enable sampling and / or publishing. |  |
| Client | Monitor Client Trigger | Use the Triggering Model if certain items are to be reported only if some other item triggers. Use proper monitoring mode for these items. Use SetTriggering Service to link these items to the trigger item. |  |

The Subscription Service Set is composed of multiple ConformanceUnits (see Table 11). Servers may provide some aspects of this functionality, see Profiles categorized as server Profiles for details. Clients may support some aspects of this functionality, see Profiles categorized as client Profiles for details.

Table 11 – Subscription Services

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Subscription Basic | Support the following Subscription Services: CreateSubscription, ModifySubscription, DeleteSubscriptions, Publish, Republish, SetPublishingMode. |  |
| Server | Subscription Minimum 02 | Support at least 2 Subscriptions per Session. |  |
| Server | Subscription Minimum 05 | Support at least 5 Subscriptions per Session. | Subscription Minimum 02 |
| Server | Subscription Publish Min 05 | Support at least 5 Publish Service requests per Session. |  |
| Server | Subscription Publish Min 10 | Support at least 10 Publish Service requests per Session. | Subscription Publish Min 05 |
| Server | Subscription Publish Discard Policy | Respect the specified policy for discarding Publish Service requests. If the maximum number of Publish Service requests has been queued and a new Publish Service request arrives, the "oldest" Publish request has to be discarded by returning the proper error. |  |
| Server | Subscription Transfer | Support TransferSubscriptions Service to transfer a Subscription from one Session to another. |  |
| Client | Subscription Client Basic | Use the Subscription and MonitoredItem Service Set as an efficient means to detect changes of Attribute values and / or to receive Event occurrences. Set appropriate intervals for publishing, keep alive notifications and total Subscription lifetime. Supply a sufficient number of Publish requests to the Server so that Notifications can be sent whenever a publish timer expires. Acknowledge received Notifications with subsequent Publish requests. |  |
| Client | Subscription Client Republish | Evaluate the sequence number in Notifications to detect lost Notifications. Use Republish to request missing Notifications. |  |
| Client | Subscription Client Modify | Allow modification of the Subscription configuration using the ModifySubscription Service. |  |
| Client | Subscription Client Multiple | Use multiple Subscriptions to reduce the payload of individual Notifications. |  |
| Client | Subscription Client Publish Configurable | Send multiple Publish Service requests to assure that the Server is always able to send Notifications. The number of parallel Publish Service requests per Session shall be configurable. |  |

## Other Features

Table 12 describes Base features related items that can be profiled. For additional information about these items, please refer to Part  6.

Table 12 – Base Information

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Base Info Core Structure | Support Server Object, Server Capabilities; support the OPC UA AddressSpace structure. |  |
| Server | Base Info Diagnostics | Support Diagnostic Objects and Variables. |  |
| Server | Base Info Type System | Present a Type System with Data Types, Reference Types, Object Types and Variable Types. This includes custom types for any non-standard types |  |
| Server | Base Info Model Change | Support ModelChange Event and NodeVersion Property for some Nodes. |  |
| Client | Base Info Client Basic | Use OPC UA defined AddressSpace structure. Access or provide access to Server information like the Server's state, BuildInfo, capabilities, Namespace Table and Type Model. |  |
| Client | Base Info Client Type Programming | Programmatically process instances of Objects or Variables by using their type definitions. This includes custom Data Types, Object Types and Variable Types |  |
| Client | Base Info Client Change Events | Process ModelChange Events to detect changes in the Server's AddressSpace and take appropriate action. |  |
| Client | Base Info Client RefreshData Mathod | Use the RefreshData Method to receive all current samples of monitored items. |  |

Table 13 describes Security related units that can be profiled.  All of these ConformanceUnits apply equally to both Clients and Servers, where a Client uses the related security unit and a server supports the use of it. These items are defined in detail in Part  6. It is recommended that a Server and Client support as many of these options as possible in order to achieve increased levels of interoperability. .

Table 13 – Security

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Security | Security User Name Password | User name password combination. Specific encryption of the password is required if no Message encryption is used. |  |
| Security | Security User X509 | A public/private key pair. Must be able to be adminstrativly disabled |  |
| Security | Security Certificate Validation | Certificate will be validated as specified in OPC UA Part 4. This includes among others structure and signature examination. Allowing for some validation errors to be suppressed by administration directive. |  |
| Security | Security None | A suite of algorithms that does NOT provide any security settings SymmetricSignatureAlgorithm- SymmetricEncryptionAlgorithm- AsymmetricSignatureAlgorithm- SymmetricKeyWrapAlgorithm- AsymmetricEncryptionAlgorithm- KeyDerivationAlgorithm PSha1 (http://docs.oasis-open.org/ws-sx/ws-secureconversation/200512/dk/p_sha1) DerivedSignatureKeyLength 0 |  |
| Security | Security None CreateSession ActivateSession | The CreateSession and ActivateSession service allow for a NULL/empty signiture and do not require Application Certificates or a Nonce. This Conformance Unit is only valid for select security policies. |  |
| Security | Security Basic 128Rsa15 | A suite of algorithms that uses RSA15 as Key-Wrap-algorithm and 128-Bit for encryption algorithms. SymmetricSignatureAlgorithm HmacSha1 (http://www.w3.org/2000/09/xmldsig#hmac-sha1) SymmetricEncryptionAlgorithm Aes128 (http://www.w3.org/2001/04/xmlenc#aes128-cbc) AsymmetricSignatureAlgorithm RsaSha1 (http://www.w3.org/2000/09/xmldsig#rsa-sha1) AsymmetricKeyWrapAlgorithm KwRsa15 (http://www.w3.org/2001/04/xmlenc#rsa-1_5) AsymmetricEncryptionAlgorithm Rsa15 (http://www.w3.org/2001/04/xmlenc#rsa-1_5) KeyDerivationAlgorithm PSha1 (http://docs.oasis-open.org/ws-sx/ws-secureconversation/200512/dk/p_sha1) DerivedSignatureKeyLength 128 |  |
| Security | Security Basic 256 | A suite of algorithms that 256-Bit for encryption algorithms SymmetricSignatureAlgorithm HmacSha1 (http://www.w3.org/2000/09/xmldsig#hmac-sha1) SymmetricEncryptionAlgorithm Aes256 (http://www.w3.org/2001/04/xmlenc#aes256-cbc) AsymmetricSignatureAlgorithm RsaSha1 (http://www.w3.org/2000/09/xmldsig#rsa-sha1) AsymmetricKeyWrapAlgorithm KwRsaOaep (http://www.w3.org/2001/04/xmlenc#rsa-oaep-mgf1p) AsymmetricEncryptionAlgorithm RsaOaep (http://www.w3.org/2001/04/xmlenc#rsa-oaep) KeyDerivationAlgorithm PSha1 (http://docs.oasis-open.org/ws-sx/ws-secureconversation/200512/dk/p_sha1) DerivedSignatureKeyLength 192 |  |
| Security | Security Administration | Allow configuration of the following Security related items. * select the proper User identification policy (User Name/Password or X509 or Kerberos) * enable/disable the Security Policy "None" or other policies * set the permitted certification authorities * define how to react to unknown Certificates |  |
| Security | Security Administration - XML Schema | Support the OPC defined xml schema for importing and export security configuration information. |  |
| Security | Security Certificate Administration | Allow the plant administrator to assign a plant specific instance Certificate and allow configuration of a plant specific CA for acceptance of Certificates |  |

Table 14 describes Protocol and encoding related features that can be profiled.  These features are defined in detail in Part  6. It is recommended that a Server and Client support as many of these options as possible for greatest interoperability.

Table 14 – Protocol and Encoding

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Protocol Configuration | Allow administration of the Endpoints and the port number used by the Endpoints. |  |
| Transport | Protocol TCP Binary UA Security | Support the UA TCP transport protocol with UA Binary Encoding and with UA Secure Conversation. |  |
| Transport | Protocol Soap Xml WS Security | Support "SOAP/HTTP" transport with XML Encoding and with WS Secure Conversation. |  |
| Transport | Protocol Soap Binary WS Security | Support "SOAP/HTTP" transport with UA Binary Encoding and with WS Secure Conversation. |  |

Table 15 describes Address Space Model information related items that can be profiled.  The details of these model items is defined in Part  3 and Part  5

Table 15 – Address Space Model

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Address Space Base | Support the NodeClasses with their Attributes and behaviour as defined in OPC UA Part 3. This includes for instance: Object, Object Type, Variable, Variable Type, References and Data Type |  |
| Server | Address Space Complex Datatypes | Support ComplexData with Data Dictionary. |  |
| Server | Address Space Method | Support Method Nodes. |  |
| Client | Address Space Client Base | Use and understand the NodeClasses with their Attributes and behaviour as defined in OPC UA Part 3. This includes for instance: Object, Object Type, Variable, Variable Type, References and Data Type |  |

Table 16 describes Data Access information model related items that can be profiled.  The details of this model are defined in Part  8.

Table 16 – Data Access

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Data Access DataItems | Provide Variables of DataItemType or one of its subtypes. Support the StatusCodes specified in the DataAccess part of the OPC UA specification. Support of optional Properties (e.g. "InstrumentRange") shall be verified during certification testing and will be shown in the Certificate. |  |
| Server | Data Access Analog | Support AnalogItemType Variables with corresponding Properties. |  |
| Server | Data Access PercentDeadBand | Support PercentDeadband filter when monitoring AnalogItemType Variables. |  |
| Server | Data Access Semantic Changes | Support semantic changes of AnalogItemType items (EURange Property and/or EngineeringUnits Property). Support semantic change StatusCode bits where appropriate. |  |
| Server | Data Access TwoState | Support TwoStateDiscreteType Variables with corresponding Properties. |  |
| Server | Data Access MultiState | Support MultiStateDiscreteType Variables with corresponding Properties. |  |
| Client | Data Access Client Basic | Understand the DataAccess Variable Types. Make use of the standard Properties if applicable. |  |
| Client | Data Access Client Deadband | Use PercentDeadband to filter value changes of AnalogItemType Variables. |  |
| Client | Data Access Client SemanticChange | Recognize the semantic change bit in the StatusCode while monitoring items and take proper action. Typically, the Client has to re-read Properties that define type-specific semantic like the EURange and EngineeringUnit Properties. |  |

Table 17 describes Alarm and Conditions information model related items that can be profiled.  The details of this model are defined in Part  9

Table 17 – Alarm & Conditions

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
|  | Undefined | No Conformance Units defined |  |

Table 18 describes Historical Data Access information model related items that can be profiled.  The details of this model are defined in Part  11.

Table 18 – Historical Access

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
|  | Undefined | No Conformance Units defined |  |

Table 19 describes Command information model related items that can be profiled.

Table 19 – Programs

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
|  | Undefined | No Conformance Units defined |  |

Table 20 describes Auditing related items that can be profiled.

Table 20 – Auditing

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Auditing Base | Support AuditEvents. The list of supported AuditEvents shall be verified during certification testing and will be shown in the Certificate. |  |
| Client | Auditing Client Audit ID | Client support generating AuditEvents ids and providing them to Servers |  |
| Client | Auditing Client Subscribes | The Client supports subscribing for audit Events and storing /processing them in a secure manner. |  |

Table 21 describes Redundancy related items that are profiled.

Table 21 – Redundancy

| Category | Title | Description | Derived |
| --- | --- | --- | --- |
| Server | Redundancy Server | Support Server-based redundancy. |  |
| Server | Redundancy Server Transparent | Support transparent Server redundancy. |  |
| Client | Redundancy Client | Client supports Client redundancy. Clients that support Client redundancy can fail over to another Client (requires some out of band communication) |  |
| Client | Redundancy Client Switch | Clients supporting this ConformanceUnit monitor the redundancy status for non-transparent redundancy Servers and switch to the backup Server when they recognize a change. |  |

# Profiles

## Overview

This section includes a listing of the categories that a Profile can be grouped into, a list of named Profiles and the detailed listing of each Profile including directly defined ConformanceUnits and any sub Profiles that are included in the Profile.

## Profile List

Table 22 lists Profiles. The Profile table is ordered by Profile category and then alphabetically by the name of the Profile.  The table includes a list of categories the profile is associated with and a URI.  The URI can be used to access the OPC Foundation WEB site and obtain additional information about the Profile. This URI is also included in the SoftwareCertificate associated with the Profile.  The OPC Foundation WEB site may also contain additional categories, which are specific to OPC Foundation generated companion specifications, but not part of the OPC UA core specifications.

An application (Client or Server) shall implement all of the ConformanceUnits in a Profile, to be compliant with the Profile. Some Profiles contain optional ConformanceUnits. An optional ConformanceUnit means that an application has the option to not support the ConformanceUnit. However, if supported, the application shall pass all tests associated with the ConformanceUnit. For example, some ConformanceUnits require specific information model items to be available. They are, therefore, listed as optional in order to allow for the information model items to be omitted. For example, if a Server desires to be listed as supporting the optional ConformanceUnit then it shall include any required information model items in the configuration provided for certification testing. The support for optional ConformanceUnits is described in the certificate that is generated by the associated testing.

Table 22 – Profile List

| Profile | Description | Related category | URI |
| --- | --- | --- | --- |
| AddressSpace Lookup Client Facet | includes all ConformanceUnits needed to navigate through the AddressSpace. | Client | http://opcfoundation.org/UA-Profile/Client/AddressSpaceLookup |
| Advanced Type Programming Client Facet | specifies the behaviour of Clients when processing instances based on Type Definitions. | Client | http://opcfoundation.org/UA-Profile/Client/TypeProgramming |
| Attribute Read Client Facet | includes ConformanceUnits needed to read Attribute values of Nodes. | Client | http://opcfoundation.org/UA-Profile/Client/AttributeRead |
| Attribute Write Client Facet | includes ConformanceUnits needed to write Attribute values of Nodes. | Client | http://opcfoundation.org/UA-Profile/Client/AttributeWrite |
| Auditing Client Facet | includes ConformanceUnits for the purpose of Auditing. | Client | http://opcfoundation.org/UA-Profile/Client/Auditing |
| Base Client Behaviour Facet | specifies behavioural aspects that Clients shall follow for best use by operators and administrators. These aspects can only be tested in a test lab. They are best practice guidelines. | Client | http://opcfoundation.org/UA-Profile/Client/Behaviour |
| Core Client Facet | defines core functionality for Clients. This facet includes the core functions for Security and Session handling. | Client | http://opcfoundation.org/UA-Profile/Client/Core |
| DataAccess Client Facet | defines ConformanceUnits needed for utilizing the DataAccess Information Model. | Client | http://opcfoundation.org/UA-Profile/Client/DataAccess |
| DataChange Subscriber Client Facet | includes ConformanceUnits needed when monitoring Attribute values for data change. | Client | http://opcfoundation.org/UA-Profile/Client/DataChangeSubscriber |
| Discovery Client Facet | includes ConformanceUnits for discovery of Servers and their Endpoints. | Client | http://opcfoundation.org/UA-Profile/Client/Discovery |
| Event Subscriber Client Facet | includes ConformanceUnits needed when subscribing for Event Notifications. | Client | http://opcfoundation.org/UA-Profile/Client/EventSubscriber |
| Method Client Facet | includes ConformanceUnits needed to call Methods. | Client | http://opcfoundation.org/UA-Profile/Client/Method |
| Node Management Client Facet | includes the ConformanceUnits that Clients will have to use to configure the AddressSpace of an OPC UA Server through OPC UA Node Management Service Set. | Client | http://opcfoundation.org/UA-Profile/Client/NodeManagement |
| Redundancy Switch Client Facet | specifies the ability to monitor the redundancy status in non-transparent redundancy Servers and switch to the backup Server when they recognize a change in status. | Client | http://opcfoundation.org/UA-Profile/Client/RedundancySwitch |
| Redundant Client Facet | defines ConformanceUnits that redundant Clients have to provide. | Client | http://opcfoundation.org/UA-Profile/Client/Redundancy |
| SecurityPolicy - Basic128Rsa15 | defines a Security Policy for medium to highly secure configurations. | Security | http://opcfoundation.org/UA/SecurityPolicy#Basic128Rsa15 |
| SecurityPolicy - Basic256 | defines a Security Policy for configurations with high security needs. | Security | http://opcfoundation.org/UA/SecurityPolicy/Basic256 |
| SecurityPolicy - None | defines a policy used for configurations with lowest security needs. This Security Policy can affect the behaviour of the CreateSession and Activate Session services. It also results in a SecureChannel which has no Channel Security | Security | http://opcfoundation.org/UA/SecurityPolicy#None |
| Auditing Server Facet | specifies Auditing functionality for Servers. | Server | http://opcfoundation.org/UA-Profile/Server/Auditing |
| Base Server Behaviour Facet | specifies behavioural aspects that Servers shall follow for best use by operators and administrators. These aspects can only be tested in a test lab. They are best practice guidelines. | Server | http://opcfoundation.org/UA-Profile/Server/Behaviour |
| Basic DataChange Subscription Server Facet | includes the ConformanceUnits when supporting basic DataChange Subscription functionality. | Server | http://opcfoundation.org/UA-Profile/Server/BaseDataChangeSubscription |
| Basic Event Subscription Server Facet | defines the basic functionality when supporting basic Subscription functionality for Events. | Server | http://opcfoundation.org/UA-Profile/Server/BaseEventSubscription |
| Client Redundancy Facet | includes the ConformanceUnits to support redundant Clients. | Server | http://opcfoundation.org/UA-Profile/Server/ClientRedundancy |
| ComplexType Server Facet | defines a set of ConformanceUnits that define Server specific handling of Complex Types. | Server | http://opcfoundation.org/UA-Profile/Server/ComplexTypes |
| Core Server Facet | defines core functionality for all Servers. This facet identifies Discovery, Session, AddressSpace and Viewing capabilities. | Server | http://opcfoundation.org/UA-Profile/Server/CoreFacet |
| DataAccess Server Facet | includes ConformanceUnits that define the handling of the DataAccess Information Model. | Server | http://opcfoundation.org/UA-Profile/Server/DataAccess |
| Embedded UA Server | focus is for devices with more than 50 Megabyte memory and with CPUs comparable to Intel 486 processor. | Server | http://opcfoundation.org/UA-Profile/Server/EmbeddedUA |
| Enhanced DataChange Subscription Server Facet | defines the typical DataChange Subscription functionality to be provided by OPC UA Servers. | Server | http://opcfoundation.org/UA-Profile/Server/EnhancedDataChangeSubscription |
| Low End Embedded Device Server | defines a Profile which is appropriate mainly for small devices with limited resources, including the inability to include a security infrastructure. It is expected that most applications will use gateways that enhance the functionality of these Servers to Embedded or Standard UA Server Profiles. | Server | http://opcfoundation.org/UA-Profile/Server/LowEndEmbeddedDevice |
| Method Server Facet | specifies the Method handling functionality for Servers. | Server | http://opcfoundation.org/UA-Profile/Server/Methods |
| Node Management Server Facet | includes the ConformanceUnits when supporting configuration of the AddressSpace through the OPC UA Node Management Service Set. | Server | http://opcfoundation.org/UA-Profile/Server/NodeManagement |
| Redundancy Transparent Server Facet | includes the ConformanceUnits for Servers with transparent redundancy. | Server | http://opcfoundation.org/UA-Profile/Server/TransparentRedundancy |
| Redundancy Visible Server Facet | includes the ConformanceUnits for visible redundancy. | Server | http://opcfoundation.org/UA-Profile/Server/VisibleRedundancy |
| Standard UA Server | defines the most common functionality for OPC UA Servers. | Server | http://opcfoundation.org/UA-Profile/Server/StandardUA |
| SOAP-HTTP WS-SC UA Binary | defines a Stack Profile which represents the combination of HTTP/SOAP 1.2, WS-SecureConversation 1.3 and UA Binary 1.0 encoding. | Transport | http://opcfoundation.org/UA-Profile/Transport/soaphttp-wssc-uabinary |
| SOAP-HTTP WS-SC UA XML | defines a Stack Profile which represents the combination of HTTP/SOAP 1.2, WS-SecureConversation 1.3 and UA XML 1.0 encoding. | Transport | http://opcfoundation.org/UA-Profile/Transport/soaphttp-wssc-uaxml |
| SOAP-HTTP WS-SC UA XML-UA Binary | defines a Stack Profile represents the combination of HTTP/SOAP 1.2, WS-SecureConversation 1.3 and UA XML 1.0 and UA Binary 1.0 encodings. | Transport | http://opcfoundation.org/UA-Profile/Transport/soaphttp-wssc-uaxml-uabinary |
| UA-TCP UA-SC UA Binary | defines a Stack Profile which represents the combination of UA TCP 1.0, UA SecureConversation 1.0 and UA Binary 1.0 encoding. | Transport | http://opcfoundation.org/UA-Profile/Transport/uatcp-uasc-uabinary |

The contents of each of the listed Profiles will be described in tabular form in a separate section.  Each table may contain references to additional Profiles and or ConformanceUnits. If a Profile is referenced it means that it is completely included.  The ConformanceUnits are referenced using their name and conformance group.  For the details of the ConformanceUnit the reader should examine the ConformanceUnit details in the appropriate conformance group section.

## Conventions for Profile definitions

Profiles are composed of ConformanceUnits. More than one Profile may contain the same ConformanceUnit. Profiles may recursively contain other Profiles

Profiles have the following naming conventions:

- Profiles intended for OPC UA Servers contain the term Server in their titles,

- Profiles intended for OPC UA Clients contain the term Client in their titles

- The term Facet in the title of a Profile indicates that this Profile is expected to be part of another larger Profile or concerns a specific aspect of OPC UA.  Profiles with the term Facet in their title are expected to be combined with other Profiles to define the complete functionality of an OPC UA Server or Client.

## Applications

A vendor that is developing a UA application, whether it is a Server application or a Client application, will review the list of available profiles.  From this list the vendor will select the profiles that include the functionality required by the application.  Typically this will be multiple Profiles. Conformance to a single Profile may not yield a complete application. In most cases multiple Profiles are needed to yield a useful application. All servers and clients shall support at least a core Profile (Core Server Facet or Core Client Facet) and at least one Transport Profile

For example an HMI Client application may choose to support the “Core Client Facet” Profile, the “UA-TCP UA-SC UA Binary” Profile, the “Data Access Client Facet” Profile, the “DataChange Subscriber Client Facet“ Profile and the “Attribute Write Client Facet” Profile. If the Client is to be lab tested then it would also support “Base Client Behaviour” Profile.   This list of Profiles would allow the Client to communicate with an OPC UA Server using UA-TCP/UA Security/UA binary.  It would be able to subscribe data, write data and would support the DA data model.  It would also follow the best practice guideline for behaviour. illustrates the Profile hierarchy that this application may contain: This figure is just an illustration and the represented Profiles may change.

Figure 2 - HMI Client Sample

Another example is an embedded device OPC UA Server application that may choose to support “Embedded UA Server” Profile and the “DataAccess Server Facet” Profile. This device would be a low-end device that would support UA-TCP, UA-Security, UA Binary encoding, data subscriptions and the DA data model. It may not support the optional attribute write. Figure 3 illustrates the hierarchy that this application may contain: This figure is just an illustration and the represented Profiles may change.

Figure 3 - Embedded Server Sample

Another simple system Server application may choose to support: “Standard UA Server” Profile and the “DataAccess Server Facet” Profile. If the Server is to be lab tested then it would also support “Base Server Behaviour” Profile. This device would be a mid-level OPC UA Server that would support all that the embedded server in the previous example supported and it would add support for an enhance level of the subscription service and support for writes. Figure 4 illustrates the hierarchy that this application may contain: This figure is just an illustration and the represented Profile may change.

Figure 4 - Standard UA Server Sample

If the example HMI Client were to connect to either of the example Servers, it may have to adjust its behaviour based on the profile reported by the respective servers.  If the HMI Client were communicating with the embedded device it would not be able to perform any write operations.  It may also have to limit the number of subscription or sessions based on the performance limits  of the server.  If the HMI client connected to the Standard Server it would be able to open additional windows, have higher limits on performance related items and it would be able to allow writes.

If an End User were examining OPC UA Servers or Clients for purchase the user would examine the Profiles supported by each product. They would also check that the Profiles that describe the functionality they desire are available from both the OPC UA Server and OPC UA Client that they wish to purchase.  The Profile and ConformanceUnit descriptions are available via the OPC Foundation WEB site.  The Profiles supported by OPC UA Clients and Servers and the level of testing that the OPC UA Applications have undergone are also available on the OPC Foundation WEB site.  End Users are encouraged to use the OPC Foundation WEB Site to review OPC UA products before purchasing them.

## Profile Tables

### Introduction

The following sections describe the Profiles in a tabular format.

Each table contains three columns. The first column is a description of the conformance group that the ConformanceUnit is part of.  This allows the reader to easily find the ConformanceUnit. This column may also state “Profile” in which case the listed item is not a ConformanceUnit, but an include Profile. The second column is a brief description of the ConformanceUnit or included Profile.  The last column indicates if the ConformanceUnit is optional or required.

### AddressSpace Lookup Client Facet

Table 23 describes the details of the AddressSpace Lookup Client Facet Profile. This Profile includes all ConformanceUnits needed to navigate through the AddressSpace.

Table 23 – AddressSpace Lookup Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Address Space Model | Address Space Client Base | False |
| Attribute Services | Attribute Client Read Base | False |
| Base Information | Base Info Client Basic | False |
| Base Information | Base Info Client Change Events | True |
| View Services | View Client Basic Browse | False |
| View Services | View Client Basic ResultSet Filtering | False |
| View Services | View Client RegisterNodes | True |
| View Services | View Client TranslateBrowsePath | True |

### Advanced Type Programming Client Facet

Table 24 describes the details of the Advanced Type Programming Client Facet Profile. This Profile specifies the behaviour of Clients when processing instances based on Type Definitions.

Table 24 – Advanced Type Programming Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Address Space Model | Address Space Client Base | False |
| Base Information | Base Info Client Basic | False |
| Base Information | Base Info Client Type Programming | False |
| View Services | View Client TranslateBrowsePath | False |

### Attribute Read Client Facet

Table 25 describes the details of the Attribute Read Client Facet Profile. This Profile includes ConformanceUnits needed to read Attribute values of Nodes.

Table 25 – Attribute Read Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Attribute Services | Attribute Client Read Base | False |
| Attribute Services | Attribute Client Read Complex | True |
| Attribute Services | Attribute Client Read with proper Encoding | True |

### Attribute Write Client Facet

Table 26 describes the details of the Attribute Write Client Facet Profile. This Profile includes ConformanceUnits needed to write Attribute values of Nodes.

Table 26 – Attribute Write Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Attribute Services | Attribute Client Write Base | False |
| Attribute Services | Attribute Client Write Complex | True |
| Attribute Services | Attribute Client Write Quality & TimeStamp | True |

### Auditing Client Facet

Table 27 describes the details of the Auditing Client Facet Profile. This Profile includes ConformanceUnits for the purpose of Auditing.

Table 27 – Auditing Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Profile | Event Subscriber Client Facet | False |
| Auditing | Auditing Client Audit ID | False |
| Auditing | Auditing Client Subscribes | False |

### Base Client Behaviour Facet

Table 28 describes the details of the Base Client Behaviour Facet Profile. This Profile specifies behavioural aspects that Clients shall follow for best use by operators and administrators. These aspects can only be tested in a test lab. They are  best practice guidelines.

Table 28 – Base Client Behaviour Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Discovery Services | Discovery Client Configure Endpoint | False |
| Security | Security Administration | False |
| Security | Security Administration - XML Schema | False |
| Security | Security Certificate Administration | False |
| Session Services | Session Client Auto Reconnect | True |
| Subscription Services | Subscription Client Multiple | False |
| Subscription Services | Subscription Client Publish Configurable | False |

### Core Client Facet

Table 29 describes the details of the Core Client Facet Profile. This Profile defines core functionality for Clients. This facet includes the core functions for Security and Session handling.

Table 29 – Core Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Profile | SecurityPolicy - Basic128Rsa15 | False |
| Profile | SecurityPolicy - None | False |
| Security | Security Administration | False |
| Security | Security User Name Password | False |
| Security | Security User X509 | False |
| Session Services | Session Client Base | False |
| Session Services | Session Client Cancel | True |
| Session Services | Session Client Detect Shutdown | False |
| Session Services | Session Client General Service Behaviour | False |
| Session Services | Session Client Impersonate | True |
| Session Services | Session Client KeepAlive | False |
| Session Services | Session Client Renew NodeIds | True |

### DataAccess Client Facet

Table 30 describes the details of the DataAccess Client Facet Profile. This Profile defines ConformanceUnits needed for utilizing the DataAccess Information Model.

Table 30 – DataAccess Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Address Space Model | Address Space Client Base | False |
| Attribute Services | Attribute Client Read Base | False |
| Attribute Services | Attribute Client Read Complex | False |
| Attribute Services | Attribute Client Read with proper Encoding | True |
| Data Access | Data Access Client Basic | False |
| Data Access | Data Access Client Deadband | True |
| Data Access | Data Access Client SemanticChange | True |

### DataChange Subscriber Client Facet

Table 31 describes the details of the DataChange Subscriber Client Facet Profile. This Profile includes ConformanceUnits needed when monitoring Attribute values for data change.

Table 31 – DataChange Subscriber Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Monitored Item Services | Monitor Client by Index | False |
| Monitored Item Services | Monitor Client Modify | True |
| Monitored Item Services | Monitor Client Trigger | True |
| Monitored Item Services | Monitor Client Value Change | False |
| Subscription Services | Subscription Client Basic | False |
| Subscription Services | Subscription Client Modify | True |
| Subscription Services | Subscription Client Multiple | True |
| Subscription Services | Subscription Client Republish | False |

### Discovery Client Facet

Table 32 describes the details of the Discovery Client Facet Profile. This Profile includes ConformanceUnits for discovery of Servers and their Endpoints.

Table 32 – Discovery Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Discovery Services | Discovery Client Configure Endpoint | False |
| Discovery Services | Discovery Client Find Servers Basic | False |
| Discovery Services | Discovery Client Find Servers Dynamic | False |
| Discovery Services | Discovery Client Find Servers with URI | True |
| Discovery Services | Discovery Client Get Endpoints Basic | False |
| Discovery Services | Discovery Client Get Endpoints Dynamic | False |

### Event Subscriber Client Facet

Table 33 describes the details of the Event Subscriber Client Facet Profile. This Profile includes ConformanceUnits needed when subscribing for Event Notifications.

Table 33 – Event Subscriber Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Address Space Model | Address Space Client Base | False |
| Monitored Item Services | Monitor Client Complex Evt Filter | True |
| Monitored Item Services | Monitor Client Event Filter | False |
| Monitored Item Services | Monitor Client Events | False |
| Monitored Item Services | Monitor Client Modify | True |
| Monitored Item Services | Monitor Client Trigger | True |
| Subscription Services | Subscription Client Basic | False |
| Subscription Services | Subscription Client Modify | True |
| Subscription Services | Subscription Client Multiple | True |
| Subscription Services | Subscription Client Republish | False |
| View Services | View Client Basic Browse | False |
| View Services | View Client TranslateBrowsePath | False |

### Method Client Facet

Table 34 describes the details of the Method Client Facet Profile. This Profile includes ConformanceUnits needed to call Methods.

Table 34 – Method Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Method Services | Method Client Call | False |

### Node Management Client Facet

Table 35 describes the details of the Node Management Client Facet Profile. This Profile includes the ConformanceUnits that Clients will have to use to configure the AddressSpace of an OPC UA Server through OPC UA Node Management Service Set.

Table 35 – Node Management Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Address Space Model | Address Space Client Base | False |
| Node Management Services | Node Management Client | False |

### Redundancy Switch Client Facet

Table 36 describes the details of the Redundancy Switch Client Facet Profile. This Profile specifies the ability to monitor the redundancy status in non-transparent redundancy Servers and switch to the backup Server when they recognize a change in status.

Table 36 – Redundancy Switch Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Base Information | Base Info Client RefreshData Mathod | True |
| Redundancy | Redundancy Client Switch | False |

### Redundant Client Facet

Table 37 describes the details of the Redundant Client Facet Profile. This Profile defines ConformanceUnits that redundant Clients have to provide.

Table 37 – Redundant Client Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Base Information | Base Info Client RefreshData Mathod | True |
| Redundancy | Redundancy Client | False |

### SecurityPolicy - Basic128Rsa15

Table 38 describes the details of the SecurityPolicy - Basic128Rsa15 Profile. This Profile defines a Security Policy for medium to highly secure configurations.

Table 38 – SecurityPolicy - Basic128Rsa15

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Security | Security Basic 128Rsa15 | False |
| Security | Security Certificate Validation | False |

### SecurityPolicy - Basic256

Table 39 describes the details of the SecurityPolicy - Basic256 Profile. This Profile defines a Security Policy for configurations with high security needs.

Table 39 – SecurityPolicy - Basic256

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Security | Security Basic 256 | False |
| Security | Security Certificate Validation | False |

### SecurityPolicy - None

Table 40 describes the details of the SecurityPolicy - None Profile. This Profile defines a policy used for configurations with lowest security needs. This Security Policy can affect the behaviour of the CreateSession and Activate Session services. It also results in a SecureChannel which has no Channel Security

Table 40 – SecurityPolicy - None

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Security | Security None | False |
| Security | Security None CreateSession ActivateSession | False |

### Auditing Server Facet

Table 41 describes the details of the Auditing Server Facet Profile. This Profile specifies Auditing functionality for Servers.

Table 41 – Auditing Server Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Auditing | Auditing Base | False |

### Base Server Behaviour Facet

Table 42 describes the details of the Base Server Behaviour Facet Profile. This Profile specifies behavioural aspects that Servers shall follow for best use by operators and administrators. These aspects can only be tested in a test lab.  They are best practice guidelines.

Table 42 – Base Server Behaviour Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Discovery Services | Discovery Configuration | False |
| Protocol and Encoding | Protocol Configuration | False |
| Security | Security Administration | False |
| Security | Security Administration - XML Schema | False |
| Security | Security Certificate Administration | False |

### Basic DataChange Subscription Server Facet

Table 43 describes the details of the Basic DataChange Subscription Server Facet Profile. This Profile includes the ConformanceUnits when supporting basic DataChange Subscription functionality.

Table 43 – Basic DataChange Subscription Server Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Monitored Item Services | Monitor Basic | False |
| Monitored Item Services | Monitor Items 100 | False |
| Monitored Item Services | Monitor QueueSize_1 | False |
| Monitored Item Services | Monitor Triggering | False |
| Monitored Item Services | Monitor Value Change | False |
| Subscription Services | Subscription Basic | False |
| Subscription Services | Subscription Minimum 02 | False |
| Subscription Services | Subscription Publish Discard Policy | False |
| Subscription Services | Subscription Publish Min 05 | False |

### Basic Event Subscription Server Facet

Table 44 describes the details of the Basic Event Subscription Server Facet Profile. This Profile defines the basic functionality when supporting basic Subscription functionality for Events.

Table 44 – Basic Event Subscription Server Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Monitored Item Services | Monitor Basic | False |
| Monitored Item Services | Monitor Events | False |
| Monitored Item Services | Monitor Items 10 | False |
| Monitored Item Services | Monitor QueueSize_1 | False |
| Subscription Services | Subscription Basic | False |
| Subscription Services | Subscription Minimum 02 | False |
| Subscription Services | Subscription Publish Discard Policy | False |
| Subscription Services | Subscription Publish Min 05 | False |

### Client Redundancy Facet

Table 45 describes the details of the Client Redundancy Facet Profile. This Profile includes the ConformanceUnits to support redundant Clients.

Table 45 – Client Redundancy Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Subscription Services | Subscription Transfer | False |

### ComplexType Server Facet

Table 46 describes the details of the ComplexType Server Facet Profile. This Profile defines a set of ConformanceUnits that define Server specific handling of Complex Types.

Table 46 – ComplexType Server Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Address Space Model | Address Space Complex Datatypes | False |
| Attribute Services | Attribute Alternate Encoding | True |
| Attribute Services | Attribute Read Complex | False |
| Attribute Services | Attribute Write Complex | False |
| Monitored Item Services | Monitor Alternate Encoding | True |

### Core Server Facet

Table 47 describes the details of the Core Server Facet Profile. This Profile defines core functionality for all Servers. This facet identifies Discovery, Session, AddressSpace and Viewing capabilities.

Table 47 – Core Server Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Profile | SecurityPolicy - None | False |
| Address Space Model | Address Space Base | False |
| Attribute Services | Attribute Read | False |
| Attribute Services | Attribute Write Index | True |
| Attribute Services | Attribute Write Values | True |
| Base Information | Base Info Core Structure | False |
| Discovery Services | Discovery Find Servers Self | False |
| Discovery Services | Discovery Get Endpoints | False |
| Security | Security Administration | False |
| Security | Security User Name Password | False |
| Security | Security User X509 | False |
| Session Services | Session Base | False |
| Session Services | Session General Service Behaviour | False |
| Session Services | Session Minimum 10 Parallel | False |
| View Services | View Basic | False |
| View Services | View Minimum Continuation Point 01 | False |
| View Services | View RegisterNodes | False |
| View Services | View TranslateBrowsePath | False |

### DataAccess Server Facet

Table 48 describes the details of the DataAccess Server Facet Profile. This Profile includes ConformanceUnits that define the handling of the DataAccess Information Model.

Table 48 – DataAccess Server Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Data Access | Data Access Analog | True |
| Data Access | Data Access DataItems | False |
| Data Access | Data Access MultiState | True |
| Data Access | Data Access PercentDeadBand | True |
| Data Access | Data Access Semantic Changes | True |
| Data Access | Data Access TwoState | True |

### Embedded UA Server

Table 49 describes the details of the Embedded UA Server Profile. This Profile focus is for devices with more than 50 Megabyte memory and with CPUs comparable to Intel 486 processor.

Table 49 – Embedded UA Server

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Profile | Basic DataChange Subscription Server Facet | False |
| Profile | Low End Embedded Device Server | False |
| Profile | SecurityPolicy - Basic128Rsa15 | False |
| Base Information | Base Info Type System | False |

### Enhanced DataChange Subscription Server Facet

Table 50 describes the details of the Enhanced DataChange Subscription Server Facet Profile. This Profile defines the typical DataChange Subscription functionality to be provided by OPC UA Servers.

Table 50 – Enhanced DataChange Subscription Server Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Profile | Basic DataChange Subscription Server Facet | False |
| Monitored Item Services | Monitor Items 500 | False |
| Monitored Item Services | Monitor MinQueueSize_02 | False |
| Subscription Services | Subscription Minimum 05 | False |
| Subscription Services | Subscription Publish Min 10 | False |

### Low End Embedded Device Server

Table 51 describes the details of the Low End Embedded Device Server Profile. This Profile defines a Profile which is appropriate mainly for small devices with limited resources, including the inability to include a security infrastructure. It is expected that most applications will use gateways that enhance the functionality of these Servers to Embedded or Standard UA Server Profiles.

Table 51 – Low End Embedded Device Server

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Profile | Core Server Facet | False |
| Profile | UA-TCP UA-SC UA Binary | False |

### Method Server Facet

Table 52 describes the details of the Method Server Facet Profile. This Profile specifies the Method handling functionality for Servers.

Table 52 – Method Server Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Address Space Model | Address Space Method | False |
| Method Services | Method Call | False |

### Node Management Server Facet

Table 53 describes the details of the Node Management Server Facet Profile. This Profile includes the ConformanceUnits when supporting configuration of the AddressSpace through the OPC UA Node Management Service Set.

Table 53 – Node Management Server Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Address Space Model | Address Space Base | False |
| Base Information | Base Info Model Change | False |
| Base Information | Base Info Type System | False |
| Node Management Services | Node Management Add Node | False |
| Node Management Services | Node Management Add Ref | False |
| Node Management Services | Node Management Delete Node | False |
| Node Management Services | Node Management Delete Ref | False |

### Redundancy Transparent Server Facet

Table 54 describes the details of the Redundancy Transparent Server Facet Profile. This Profile includes the ConformanceUnits for Servers with transparent redundancy.

Table 54 – Redundancy Transparent Server Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Redundancy | Redundancy Server Transparent | False |

### Redundancy Visible Server Facet

Table 55 describes the details of the Redundancy Visible Server Facet Profile. This Profile includes the ConformanceUnits for visible redundancy.

Table 55 – Redundancy Visible Server Facet

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Redundancy | Redundancy Server | False |

### Standard UA Server

Table 56 describes the details of the Standard UA Server Profile. This Profile defines the most common functionality for OPC UA Servers.

Table 56 – Standard UA Server

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Profile | Embedded UA Server | False |
| Profile | Enhanced DataChange Subscription Server Facet | False |
| Attribute Services | Attribute Write StatusCode & TimeStamp | True |
| Base Information | Base Info Diagnostics | False |
| Discovery Services | Discovery Register | False |
| Discovery Services | Discovery Register Security | False |
| Session Services | Session Cancel | False |
| Session Services | Session Minimum 50 Parallel | False |
| View Services | View Minimum Continuation Point 05 | False |

### SOAP-HTTP WS-SC UA Binary

Table 57 describes the details of the SOAP-HTTP WS-SC UA Binary Profile. This Profile defines a Stack Profile which represents the combination of HTTP/SOAP 1.2, WS-SecureConversation 1.3 and UA Binary 1.0 encoding.

Table 57 – SOAP-HTTP WS-SC UA Binary

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Protocol and Encoding | Protocol Soap Binary WS Security | False |

### SOAP-HTTP WS-SC UA XML

Table 58 describes the details of the SOAP-HTTP WS-SC UA XML Profile. This Profile defines a Stack Profile which represents the combination of HTTP/SOAP 1.2, WS-SecureConversation 1.3 and UA XML 1.0 encoding.

Table 58 – SOAP-HTTP WS-SC UA XML

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Protocol and Encoding | Protocol Soap Xml WS Security | False |

### SOAP-HTTP WS-SC UA XML-UA Binary

Table 59 describes the details of the SOAP-HTTP WS-SC UA XML-UA Binary Profile. This Profile defines a Stack Profile represents the combination of HTTP/SOAP 1.2, WS-SecureConversation 1.3 and UA XML 1.0 and UA Binary 1.0 encodings.

Table 59 – SOAP-HTTP WS-SC UA XML-UA Binary

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Protocol and Encoding | Protocol Soap Binary WS Security | False |
| Protocol and Encoding | Protocol Soap Xml WS Security | False |

### UA-TCP UA-SC UA Binary

Table 60 describes the details of the UA-TCP UA-SC UA Binary Profile. This Profile defines a Stack Profile which represents the combination of UA TCP 1.0, UA SecureConversation 1.0 and UA Binary 1.0 encoding.

Table 60 – UA-TCP UA-SC UA Binary

| Group | Conformance Unit / Profile Title | Optional |
| --- | --- | --- |
| Protocol and Encoding | Protocol TCP Binary UA Security | False |